PREFLIGHT

Configure cockpits

REAR

Mag Control UP (1st cabin)

Gear NEUTRAL

Flaps NEUTRAL

Instructor Switches DOWN

Mags 1+2

Brake Override OFF (Back)

Parachute Removed

Straps Tied and Secure

Canopy Closed and Locked

FRONT

Mags OFF (0)

Starter Guard in place

All Switches OFF

Gear DOWN & Latched

Brakes ON

Main air supply ON

Check seatbelts

EXTERNAL

Prop Area clear

Cowl Shutters

Fuselage

Gear Struts

Oil Leaks

Cowl Latches

Flaps

Drain Fuel

Control Surfaces

Pitot cover OFF

Static Ports

Oil Cooler Cover

Fuel quantity

Oil Quantity

Pull Prop through 12 blades

(priming before pulling

through may aid starting)

ENTERING COCKPIT

Seatbelts secure

Headset connected

Rudder pedals adjusted

Canopy free

Controls free

Trim free

Clock wind and set

STARTING

Air supply Recheck

Cooling controls as required

Throttle slightly open

Fuel supply ON

Carb heat OFF

Master switch ON

Engine Instruments ON

Annunciator lights ON

Primer LEFT and pump for fuel pressure

Primer right and prime as req’d

Generator ON

Ignition ON

Press starter

Mags to BOTH (1+2)

Prime as req’d then lock

Warm up at 40%

Electrical systems as req’d

WARM UP

Monitor temps and Pressures

Set cooling controls as required

Switch on and test avionics

TAXIING

Test brakes

Test Gyros

Test Flaps

BEFORE TAKEOFF

Mag check at 70% RPM

Exercise Prop

Check primer locked

Throttle Friction SET

TRIM set for takeoff

Mags - Both (1+2)

Prop MAX RPM

Flaps UP

Check Engine guages

Align Gyros

Canopy as req’d

Seatbelts tight

Transponder to ALT

TAKEOFF

Apply FULL power

Lift nosewheel JUST clear of the ground

Climb at 85-90 Kts

Gear UP

Power to 80%, 80 cms

BEFORE LANDING

Power to 70% & 40 cms

Abeam touchdown area, GEAR   DOWN, check lights and poles

AFTER LANDING

Taxy clear of runway and STOP

FLAPS Identify and raise

Pitot heat OFF

Transponder to STBY

SHUT-DOWN

All breakers and switches OFF

Prop MAX rpm

Throttle 30%

CHT 140-150

70% RPM for 10 secs then 30 %

Mags OFF

Air valve OFF

Drain Air system

YAK 52 CHECKLISTS

Assumes solo from front seat

v1.3